# Nexus — AI-Powered Home Security System

## Project Deployment Guide

Graduation Project — Menoufia National University

Faculty of Computers and Artificial Intelligence

Academic Year 2025/2026

---

## System Requirements

### Desktop Application

| Requirement | Specification |
| --- | --- |
| Operating System | Windows 10 / Windows 11 (64-bit) |
| Processor | Intel Core i5 or equivalent, 2.0 GHz or higher |
| RAM | Minimum 4 GB (8 GB recommended) |
| Storage | Minimum 10 GB free space |
| Camera | USB webcam or built-in camera |
| Internet | Stable connection, minimum 5 Mbps upload |
| Python | Version 3.10 or higher |

### Mobile Application

| Requirement | Specification |
| --- | --- |
| Android | Version 8.0 (API level 26) or higher |
| iOS | Version 13.0 or higher |
| Internet | WiFi or mobile data |

### Hardware Device

| Component | Specification |
| --- | --- |
| Microcontroller | ESP32 DOIT DevKit V1 |
| Sensor | DHT22 (Temperature and Humidity) |
| Actuator | Servo Motor |
| Alarm | Buzzer |
| Power | 5V USB |
| Network | 2.4 GHz WiFi |

---

## Part 1 — Desktop Application Deployment

### Step 1 — Install Python

Download and install Python 3.10 or higher from:

https://www.python.org/downloads/

During installation, check the option "Add Python to PATH"

---

### Step 2 — Download the Project

Clone the repository or download the ZIP file from:

https://github.com/Mohamed3taa/Home_Security

---

### Step 3 — Install Dependencies

Open a terminal (CMD) in the project folder and run:

pip install -r DesktopApp/codes/Requirements.txt

Dependencies installed:

opencv-python

paho-mqtt

PyQt6

cryptography

qrcode

pillow

google-genai

firebase-admin

cloudinary

---

### Step 4 — Configure Firebase Credentials

1. Go to Firebase Console: https://console.firebase.google.com

2. Open project home-security-f3878

3. Go to Project Settings → Service Accounts

4. Click Generate New Private Key

5. Place the downloaded JSON file in the `DesktopApp/` folder

---

### Step 5 — Configure config.py

Open `DesktopApp/codes/config.py` and fill in the following:

VIDEO_DEVICE_INDEX = 0          # Camera index (0 for default camera)
MQTT_BROKER = "YOUR_HIVEMQ_CLUSTER_URL"
MQTT_USERNAME = "YOUR_MQTT_USERNAME"
MQTT_PASSWORD = "YOUR_MQTT_PASSWORD"
GEMINI_API_KEY = "YOUR_GEMINI_API_KEY"
FIREBASE_CREDENTIALS_FILE = "YOUR_ADMINSDK_FILE.json"
TARGET_USER_ID = "YOUR_FIREBASE_USER_UID"
CLOUDINARY_CLOUD_NAME = "YOUR_CLOUDINARY_CLOUD_NAME"
CLOUDINARY_API_KEY = "YOUR_CLOUDINARY_API_KEY"
CLOUDINARY_API_SECRET = "YOUR_CLOUDINARY_API_SECRET"

---

### Step 6 — Run the Application

python DesktopApp/codes/main.py

The application window will open showing:

Live camera feed (left panel)

QR code for mobile pairing (right panel)

Event log at the bottom

---

## Part 2 — Mobile Application Deployment

### Step 1 — Install Flutter

Download and install Flutter SDK 3.x from:

https://flutter.dev/docs/get-started/install

---

### Step 2 — Install Dependencies

Open a terminal in the `home_security/` folder and run:

flutter pub get

---

### Step 3 — Configure Firebase

Place the `google-services.json` file (Android) in:

home_security/android/app/google-services.json

---

### Step 4 — Build the Application

For Android (APK):

flutter build apk --release

The APK file will be located at:

home_security/build/app/outputs/flutter-apk/app-release.apk

Install on Android device:

flutter install

---

### Step 5 — First Launch Setup

1. Open the app on your mobile device

2. Register a new account using email/password or Google Sign-In

3. Verify your email address

4. On the Home screen, tap the camera settings icon

5. Select Add Camera

6. Scan the QR code displayed on the Desktop Application

7. The live stream will start automatically

---

## Part 3 — Hardware Firmware Deployment (ESP32)

### Step 1 — Install VS Code and PlatformIO

1. Download VS Code from: https://code.visualstudio.com

2. Open VS Code → Extensions → Search for PlatformIO IDE → Install

---

### Step 2 — Open the Hardware Project

Open the `smart device/` folder in VS Code

---

### Step 3 — Configure WiFi and Firebase

Open `smart device/src/config.h` and update:

#define WIFI_SSID       "YOUR_WIFI_NAME"
#define WIFI_PASSWORD   "YOUR_WIFI_PASSWORD"
#define API_KEY         "YOUR_FIREBASE_API_KEY"
#define USER_EMAIL      "YOUR_FIREBASE_EMAIL"
#define USER_PASSWORD   "YOUR_FIREBASE_PASSWORD"
#define PROJECT_ID      "home-security-f3878"
#define DEVICE_DOC_ID   "YOUR_DEVICE_DOCUMENT_ID"

---

### Step 4 — Connect and Flash

1. Connect the ESP32 board to your PC via USB cable

2. In VS Code bottom toolbar click the Upload button (→ arrow icon)

3. Wait for the upload to complete

4. Open Serial Monitor to verify the device connected to WiFi and Firebase

---

### Step 5 — Pair with Mobile App

1. Open the mobile app

2. Go to Settings → Hardware → Add Device

3. Scan the hardware QR code

4. The device will appear in the Hardware Control screen

---

## Part 4 — Cloud Services Configuration

### Firebase Setup

| Service | Configuration |
| --- | --- |
| Authentication | Enable Email/Password and Google Sign-In providers |
| Firestore | Create database in production mode |
| Cloud Messaging | Enable for Android and iOS |

### HiveMQ Cloud Setup

1. Create a free cluster at: https://www.hivemq.com/mqtt-cloud-broker/

2. Create credentials (username and password)

3. Use the cluster URL and credentials in `config.py`

### Cloudinary Setup

1. Create a free account at: https://cloudinary.com

2. Get Cloud Name, API Key, and API Secret from the dashboard

3. Add them to `config.py`

### Google Gemini API Setup

1. Go to: https://aistudio.google.com/app/apikey

2. Create a new API key

3. Add it to `config.py`

---

## System Verification Checklist

After completing all steps, verify the following:

| Check | Expected Result |
| --- | --- |
| Desktop app opens | Camera feed visible, QR code displayed |
| Mobile app paired | Camera appears in dropdown list |
| Live stream works | Video visible in mobile app with LIVE badge |
| AI detection works | Alerts appear in notification screen |
| Hardware connected | Temperature and humidity visible in app |
| Gate control works | Servo moves when toggled from app |
| Alarm control works | Buzzer activates when toggled from app |
| Push notifications | Notification received on mobile when anomaly detected |

---

## Troubleshooting

| Problem | Solution |
| --- | --- |
| Camera not detected | Change VIDEO_DEVICE_INDEX in config.py (try 0, 1, 2) |
| MQTT connection failed | Check broker URL and credentials in config.py |
| AI not detecting | Verify GEMINI_API_KEY is valid and has quota |
| No push notifications | Check FCM token is saved in Firestore Users collection |
| ESP32 not connecting | Verify WiFi credentials in config.h |
| Stream not showing | Re-scan QR code and reconnect from mobile app |

---

*Nexus — AI-Powered Home Security System*

*Menoufia National University — 2025/2026*